# Bally Old Chicago

## Physical Setup

Level Machine – about 1% steeper than factory recommendation

Playfield Panel Post Adjustment: HARD

Ear Plug Tilt Hack

Verify Tilt Sensitivity – should be sensitive, but only one danger with each hard bump

Check Flipper Strength, Consider Glo Balls if playing too slow or can’t hit upper playfield

Ensure center eject is not easily exploitable; needs to be hard kickout. Use glo ball if needed.

Clean/wax playfield

Clean glass

## Switch Setup

5-Ball Play

Extra Balls – disable if possible (drop target Adjustment Plug)

Hi Score Adjustment Plug: make it lowest score possible, to replenish credits

Double Bonus Feature: 1,3,5 on a 5-ball game