# Space Invaders

## Physical Setup

Level Machine

Verify Tilt Sensitivity. If Tilt Ends Game, Note it on Backglass

Check coin door killing flippers.

Check Flipper Strength. Consider Glo Balls.

Install thick post/rubber to make horseshoe harder

Disable horseshoe switches if can’t make it harder.

## Hardware Setup

SW#31 – Off, 3-Ball Play.

SW#24 – Off, Center rollover scores captive ball value, does not increase

SW#23 – Off, Red Invader not stored in memory

SW#32 – Off, Blue invader inlanes separate

Horseshoe shot:

  - Hard to make: SW#6 – On, SW#7 – Off:  Special/EB are points

  - Easy to make: SW#6 – On, SW#7 – On:  EB enabled.

SW#14 – On, Pop bumpers score 1000.

Audio:  SW#29, 30, 8, 15,

Reset HSTD