# Elvira Party Monsters

## Physical Setup

Level Machine

Outlane Posts on “Hard Settings”. I.e. most open.

Verify Tilt Sensitivity

Ear Plug Tilt Hack (see photo)

Verify Volume

Verify coin door ball save or no flipper disable when coin door opened.

Clean Playfield if necessary

Clean Glass

Check Flipper Strength

Remove 1 ball, Install 2 balls only. Helps to balance lock stealing.

## Standard Adjustments

#70 Reset to Factory Default.

#55 Install 3 Ball

#11 Max EB: 0

#23 Free Play

## Feature Adjustments

#36 Jackpot = 4,000,000

#37 ELVIRA Start: ALL OFF

#40 ELVIRA Memory: Player

#44 Save Locks: YES (do NOT save locked balls at game end)

#48 Contest Mode: YES.