# Creature from the Black Lagoon (CFTBL)

## Physical Setup

Level Machine: factory recommended steepness

Outlane Posts on “Medium Settings”. I.e. MIDDLE position

Ear Plug Tilt Hack

Verify Tilt Sensitivity – should be sensitive, but only one danger with each hard bump

Verify no flipper disable when coin door opened. Fix if needed.

Check Flipper Strength, especially left ramp during multiball

Clean Playfield if necessary

Clean Glass

Verify Volume

## Software Setup

U9.03 Install Medium.

U9.07 Install 3 Ball

U9.10 Install Novelty.  (No EB, Special is points)

## Adjustments

Verify A1-03, A1-04 No Extra Balls

A1.26 Tournament Play YES

A1.33 Game Restart: Never

A2.07 Ball Save Timer: 3 Seconds

A3-17: Free Play: YES

## Posted Notes

None