# Fish Tales

## Physical Setup

Level Machine

Outlane Posts on “Hard Settings”. I.e. most open.

Verify Tilt Sensitivity

Verify coin door ball save or no flipper disable when coin door opened.

Verify lighting flippers are installed

## Software Setup

Reset to Factory Default.

Tournament/Competition Mode

Standard Adjustments:

3 Ball (A.1-01)

ONE Tilt Warning (A.1-02)

Extra Ball Limit: NO EXTRA BALLS. (A.1-03)

Special award: POINTS. (A.1-15)

Tournament play: YES (A.1-26)

Game Restart: NO (A.1-33)

Disable coin door switch if necessary.

Feature Adjustments:

Lock Difficulty: Hard (A.2-10)

Ball Save Time: 4 (A.2-11)

Castle Lock Timer: 10 seconds (A.2-12) (or less, decrease from default)

0 pre-lit coffin (A.2-15)

Free Play: ON (A.3-17)

Bats to 20 (A.2-32)

Family Mode (A.2-43)