# The Addams Family

## Physical Setup

Level Machine, slightly steeper than factory recommendation

Outlane Posts on “Hard Settings”. I.e. most open.

Ear Plug Tilt Hack

Verify Tilt Sensitivity – should be sensitive, but only one danger with each hard bump

Verify no flipper disable when coin door opened. Fix if needed.

Check Flipper Strength, verify ramps are hittable with ease

Clean Playfield if necessary

Clean Glass

Verify Volume

## Software Setup - Utilities

U9.03 Install Hard.

U9.07 Install 3-ball

U9.10 Install Novelty.  (No EB, Special is points)

## Adjustments, after Utilities

Verify A1-04 No Extra Balls

A1.26 Tournament Play YES

A2.23 Quick Multiball Memory YES

A3-17: Free Play: YES

## Posted Notes

None