# Twilight Zone

Note: If any items are setup differently than as described here to address issues with the machine or these listed settings, that’s fine; just please make a note of the differences on this sheet somewhere.

## Physical Setup

Verify Machine Level Machine

Known power issues for flippers and kickouts: troubleshoot and fix

Outlane Posts on “Hard Settings”. I.e. most open.

Make slingshots sensitive

Install Ear Plug Tilt Hack

Verify Tilt Sensitivity – should be sensitive, but only one danger with each hard bump

Verify no flipper disable when coin door opened. Fix if needed.

Check Flipper Strength, verify ramps are hittable with ease

Clean Playfield if necessary

Clean Glass

Verify Volume

Remove ONE BALL from game, to leave only TWO in GUMBALL Machine

## Software Setup

U9.03 Install Medium.

U9.07 Install 3-ball

U9.10 Install Novelty.  (No EB, Special is points)

A3.06 Units/Credits: 1 - Pricing: 25 cent play

## Standard Adjustments

A1.26 Tournament Play YES

A1.33 Game Restart: Never

A2.01: OFF No buy-in

## Feature Adjustments

A1-04: Max extra balls per ball in play to 0

A?-??: Number of balls in Gumball machine: 2 (not sure this setting is in ROM, or just have machine detect one less ball in machine after power-cycling)

## Posted Notes

None