# Congo

## Physical Setup

Level Machine

Outlane Posts on “Hard Settings”. I.e. most open.

Verify Tilt Sensitivity

Verify coin door ball save or no flipper disable when coin door opened.

Check Flipper Strength

## Software Setup

U8 Reset to Factory Default.

U9.08 Install 3-ball

U9.10 Install Novelty.

A3-17: Free Play: ON

Standard Adjustments:

A1-02: 2 tilt warnings

A1-03, A1-04 No Extra Balls

A1-15, Special worth points.

A1.33 Game Restart: Never

No buy-in

Disable coin door switch if necessary.

A1.26 Tournament Mode

Feature Adjustments:

A2.01 Ball Save: 1

A2.02 Ball Save Time: 2

A2.08 Kickback Difficulty: Hard

A2.09 Multiball Difficulty:

A2.17 End Game Ball Release: Yes