# Jackbot

## Physical Setup

Level Machine slightly steeper than factory recommendation

Outlane Posts on “Hard Settings”. I.e. most open.

Ear Plug Tilt Hack

Verify Tilt Sensitivity – should be sensitive, but only one danger with each hard bump

Verify no flipper disable when coin door opened. Fix if needed.

Check Flipper Strength

Clean Playfield if necessary

Clean Glass

Verify Volume

## Software Setup - Utilities

U9.03 Install Medium.

U9.07 Install 3-ball

U9.10 Install Novelty.  (No EB, Special is points)

## Adjustments, after Utilities

Verify A1-04 No Extra Balls

A1.26 Tournament Play YES

A1.33 Game Restart: Never

A2.01: No buy-in

A2.16: Cheating Allowed: NO

A2.19: Free Ride Time: 0

A3-17: Free Play: YES

## Posted Notes

Cheating feature disabled. Ball saver disabled.