# Guns N Roses

## Physical Setup

Level Machine, slightly steeper than factory recommendation

Outlane Posts on “Hard Settings”. I.e. most open.

Ear Plug Tilt Hack

Verify Tilt Sensitivity – should be sensitive, but only one danger with each hard bump

Verify no flipper disable when coin door opened. Fix if needed.

After #34 adjustment, Check Flipper Strength, verify ramps are hittable with ease from stopped ball

Clean Playfield if necessary

Clean Glass

Verify Volume

## Software Setup

#7 Install HARD.

Standard Adjustments after Software Setup

#5 Limit Freegame 0

#6 Limit Extra Balls 0

#15 Balls Per Game 3

#32 Free Play ON

#34 Coil Pulse Power to HARD

#36 Coils Pulse: HARD

#37 Ramp Timer: 5

#39 Lock Feature: Hard

#40 Skyway: Extra Hard

## Posted Notes

None