# Sega Starship Troopers

## Physical Setup

Level Machine, much steeper than factory recommendation

Outlane Posts on “Hard Settings”. I.e. most open.

Remove outlane rubbers, both on post above inlane and movable outlane post

Ear Plug Tilt Hack

Verify Tilt Sensitivity – should be sensitive, but only one danger with each hard bump

Verify no flipper disable when coin door opened. Fix if needed.

After #32 adjustment, Check Flipper Strength, verify ramps are hittable with ease from stopped ball

Clean Playfield if necessary

Clean Glass

Verify Volume

Verify Brain Bug working correctly and registering hits. If not, use #50 to disable it.

## Software Setup

#7 Install HARD.

## Standard Adjustments after Software Setup

#5 OFF – No Extra Balls

#12 3 Balls per game

#28 YES Free Play

#32 Coil Pulse Power to HARD

#36 NO Restart Game

#39 PAPA Tournament Style.

#44 EXHARD Multiball Restart

#46 HARD Multiball Criterion

#47 EXHARD Orbit Criteria

#50 Brain Bug – some SST have issues registering hits. If that’s the case, disable it.