# Stern Terminator 3

## Physical Setup

Level Machine: 1% steeper than factory recommended

Outlane Posts on “Hard Settings”. I.e. fully open position

Remove rubber from movable outlane post, keep it on post between inlane/outlane

Ear Plug Tilt Hack

Verify Tilt Sensitivity – should be sensitive, but only one danger with each hard bump

Verify no flipper disable when coin door opened. Fix if needed.

Check Flipper Strength, especially for hitting ramps

Clean Playfield if necessary

Clean Glass

Verify Volume

## Install Menu (in order)

Install Hard

Install Free Play Competition.

Install Novelty

## Standard Adjustments, verify or update AFTER “Install Menu”

#06 Special Limit: POINTS.

#09 Extra Ball Limit: NO EXTRA BALLS.

#14 Balls per Game: 3

#30 Free Play: YES

#33 Coil Power: HARD (verify machine plays ok)

#35 Game Restart: NO

#38 Freeze Save Time: 3

#40 Timed Plunger OFF

#41 Flipper Ball Launch DISABLED

#42 Coin Door Ball Saver: YES

#43 Competition Mode: YES

#44 Consolation Ball: NO

#48 Player Competition: NO

Feature Adjustments:

#03 Start Multiball Diff: EXHARD

#05 Kickback Relight Diff: EXHARD

#09 Disable RPG: YES.

#11 Adult Content: NO.

Disable/Reduce Multiball Ball Saver if possible.

Posted Notes:

- RPG Disabled – 5M Points. [verify]