# Stern Avengers

## Physical Setup

Level Machine, much steeper than factory recommendation

Outlane Posts on “Hard Settings”. I.e. most open. Remove rubbers from outlane posts.

Ear Plug Tilt Hack

Verify Tilt Sensitivity – should be sensitive, but only one danger with each hard bump

Verify coin door ball save or no flipper disable when coin door opened.

After #43 adjustment, Check Flipper Strength, ensure ramp are easily hittable from stopped ball.

Clean Playfield if necessary

Clean Glass

Verify Volume

## Install Menu

- Install Competition.

## Standard Adjustments, verify or update AFTER “Install Competition”

- #12 Special Limit: NO SPECIALS.

- #16 Extra Ball Limit: NO EXTRA BALLS.

- #21 Balls per Game: 3

- #38 Free Play: YES

- #43 Coil Power: HARD (verify machine plays ok)

- #45 Game Restart: NO

- #48 Ball Save Time: 3

- #49 Timed Plunger OFF

- #50 Flipper Ball Launch OFF

- #51 Coin Door Ball Saver: YES

- #53 Consolation Ball: NO

- #57 Player Competition: YES

- #63 Lost Ball Recovery: NO

## Feature Adjustments – Verify what’s available on adjustments menu; not in manual.

- Reduce Multiball ball save times to minimum (for the vs modes)

- Loki Multiball to Hard

- Hawkeye and Hulk modes to extra hard. Should need four left shots.

- Other modes to easy; Thor Ironman, CapAmerica, & BW should have two lights spotted at start of game

- Left kickback on extra-hard settings if possible. If not possible, disable kickback by either disconnecting switch or solenoid (if not too difficult).

## Posted Notes

- Hawkeye, Hulk, and Loki on Extra Hard

- Black Widow, Thor, Captain America, and Iron Man on Easy