# Batman 66

## Physical Setup

Verify Machine Level Machine. Increase steepness if possible.

- Outlane Posts on “Hard Settings”. I.e. most open.

- Remove rubber on outlane posts

Make slingshots extra sensitive

Install Ear Plug Tilt Hack

- Verify Tilt Sensitivity – should be quite sensitive, but still only one danger with each hard bump

- Check coin door ball save.

- Check Flipper Strength (adjust #43 setting below if needed)

- Clean/wax playfield if necessary

- Clean glass

- Verify Volume

## Install Menu

Install 3-Ball

Install Medium

Install Competition.

25 cent play

## Standard Adjustments, verify or update AFTER “Install”

#14 Special Award: POINTS.

#16 Extra Ball Limit: NO EXTRA BALLS.

#21 Balls per Game: 3

#38 Free Play: YES

#43 Coil Power: HARD (if necessary)

#45 Game Restart: NO

#48 Ball Save Time: 5

#49 Timed Plunger OFF

#50 Flipper Ball Launch OFF

#51 Coin Door Ball Saver: YES

#52 Competition Mode: YES

#53 Consolation Ball: NO

#63 Lost Ball Recovery: NO

#64 Coin Door Disable Tilt: YES

## Feature Adjustments

- Reduce Multiball ball save times to 5 seconds

- Set MB Start to Hard for first MB (one lock lit at a time)

## Posted Notes