# Stern Iron Maiden

## Physical Setup

- Level Machine – about 1% steeper than factory recommendation

- Outlane Posts on “Hard Settings”. I.e. most open.

- Adjust slingshots to be extra sensitive

- Ear Plug Tilt Hack

- Verify Tilt Sensitivity – should be sensitive, but only one danger with each hard bump

- Check coin door ball save.

- Check Flipper Strength (after #43 setting below)

- Clean/wax playfield

- Clean glass

- Verify Volume

## Install Menu

Install 3-Ball

Install Hard

Install Competition.

## Standard Adjustments, verify or update AFTER “Install”

#14 Special Award: POINTS.

#16 Extra Ball Limit: NO EXTRA BALLS.

#21 Balls per Game: 3

#38 Free Play: YES

#43 Coil Power: HARD (verify machine plays ok)

#45 Game Restart: NO

#48 Ball Save Time: 3

#49 Timed Plunger OFF

#50 Flipper Ball Launch OFF

#51 Coin Door Ball Saver: YES

#52 Competition Mode: YES

#53 Consolation Ball: NO

#57 Player Competition: NO

#63 Lost Ball Recovery: NO

#64 Coin Door Disable Tilt: YES

## Feature Adjustments

- Reduce Multiball ball save times to minimum

- Disable Secret Skill Shots

- Disable Sarcophagus (Virtual Lock)

- Disable Center Ramp (if Premium/LE)

## Posted Notes

- TBD