# Williams Scorpion

## Physical Setup

Level Machine – about 1% steeper than factory recommendation

Ear Plug Tilt Hack

Verify Tilt Sensitivity – should be sensitive, but only one danger with each hard bump

Check Flipper Strength, Consider Glo Balls if playing too slow or can’t hit upper playfield

Clean/wax playfield

Clean glass

## Switch or Software Setup

3-Ball Play. (F30=03)

Disable Extra Balls (F33=00)

Special worth points (F27=02, F34=10)

Max Credits to Max (F18 to max)

Replay Score #1=10K (F14=10,000, F28=00)

Multiball once per ball (F31=02)

50K for 20 seconds (F32=20)

Bonus X, No Memory, reset to 1x (F35=02)